# CS 203: Computer Architecture

Winter 2017, Test 1

# Points: 60 points                                                                      Time: 1 hour and 20 mins

1. (a) Computer A has an overall CPI of 1.3 and can be run at a clock rate of 600MHz. Computer B has a CPI of 2.5 and can be run at a clock rate of 750 Mhz. We have a particular program we wish to run. When compiled for computer A, this program has exactly 100,000 instructions. How many instructions would the program need to have when compiled for Computer B, in order for the two computers to have exactly the same execution time for this program?

---- 5 pts

(b) Give the equations for the power and energy consumption in a single-core CPU in terms of voltage and frequency.                                                                                                   ---- 5 pts

(c) Consider a system for a real-time application in which specific deadlines must be met. With the current speed you can execute the program in half the time. How much power and energy you save by setting the voltage and frequency to half as much?                                    ----7 pts

2. Consider the following sequence of MIPS instructions. 36. LOOP:   LW R1, 4(R2);           40. ADD R3, R3, R1;   44. LW R2, 0(R2);   48. BNE R2, R0, LOOP.

(a) 1. Write the logic function to detect data dependency hazard in a MIPS 5-stage pipeline.     2. What is the penalty between instructions 36 and 40 above with and without internal forwarding in the datapath? Explain why.                                                                      ---- 8 pts

(b) Assume internal forwarding in the datapath. 1. What is the branch penalty when a branch takes place with fixed “predict taken” and “predict not-taken” policies when the branch unit is in EX stage?

2. How does the penalty change when we move the branch unit to the ID stage?

3. Then, what is the branch penalty between instructions 44 and 48 in the above program?

---- 8 pts

(c) 1. Calculate the value of offset LOOP in the instruction 48 of the above program?              2. Without drawing the space-time diagram, determine the time that the program takes for one iteration. Assume full internal forwarding and branch unit movement                          ---- 7 pts

3. (a) Below is a list of 32-bit memory address references, given as word addresses.

1, 134, 212, 1, 135

1. For each of those references, identify the binary address, tag and the index bits given a direct-mapped cache with two-word blocks and a total cache size of eight blocks.

2. Also list if each reference is a hit or miss, assuming the cache is initially empty.  ----- 10 pts

(b) 1. What is critical word first and early restart technique to reduce the miss penalty?

Assume a 1MB L2 cache with 64-byte blocks and refill path 16bytes wide. Assume that L2 can be written with 16 byte block every 4 cycles, the time to read the first 16 bytes from memory is 120 cycles, each additional 16 byte block from memory requires 16 cycles.

2. Derive how many cycles would it take to service an L2 cache miss with and without critical word first and early restart?

----- 10 pts

3. (a) Below is a list of 32-bit memory address references, given as word addresses.

1, 134, 212, 1, 135

1. For each of those references, identify the binary address, tag and the index bits given a direct-mapped cache with two-word blocks and a total cache size of eight blocks.       a1. 6 pts    a2. 4 pts.

Answer:   Assuming the machine is word addressable, the block offset is 1 bit for 2 words/block.

There are 8 blocks. So, index is log2 8 = 3 bits.

(a) 1. Word address              Tag       Index      Offset   (a.2) Hit/Miss       Comments

1                000 …….0000         000            1                  M             Compulsory miss

134              000 …….1000         011            0                 M                  Same

212             000……. 1101         010            0                 M                  Same

1               000 …….0000         000            1                 H         Present in block 0 (000)

135             000 …….1000         011            1                 H         Present in block 3 (011)

(b) 1. What is critical word first and early restart technique to reduce the miss penalty? 4 pts

Assume a 1MB L2 cache with 64-byte blocks and refill path 16bytes wide. Assume that L2 can be written with 16 byte block every 4 cycles, the time to read the first 16 bytes from memory is 120 cycles, each additional 16 byte block from memory requires 16 cycles.

Answer: Critical word first read the requested word from main memory and load it to the cache so that the instruction can continue while the block is loaded. Early restart is to load the block normally to the cache, but supply the word to the CPU as soon as it arrives.

2. Derive how many cycles would it take to service an L2 cache miss with and without critical word first and early restart?                                                      6 pts

Answer: L2 cache penalty is 120 cycles + 3x4 = 132 cycles. With critical word first and early restart, it is 120 cycles only. It is because the early restart word is already included in the first block.

The answer is more complicated if you separate these two techniques and calculate the service time. So, credit has been given to both answers.