# CS 203: Computer Architecture

Winter 2018, Test 1

# Points: 60 points                                                                      Time: 1 hour and 20 mins

1. (a) Assume that a computer at 1 GHz frequency is running a program with 20,000 instructions. The instruction mix consists of 50% ALU, 20% load, 10% store, and 20% branch instructions, which take 1, 2, 3 and 2 cycles, respectively. Derive the CPI of the program.                                                                                                               --- 6 pts

Answer: 0.5 x 1 + 0.2 x 2 + 0.1 x 3 + 0.2 x 2 = 1.6

(b) Instead of a load-store architecture, assume that you would like to introduce a new Reg/Mem instruction that can get one operand from register and another from memory location for an arithmetic operation. What fraction of load must be eliminated for this instruction to pay off?                                                                                      ----- 8 pts

Answer: Let X be the fraction of new instructions. Then the loads and ALU instructions will also be reduced by the same fraction X.

New instructions count Inew = (X for Reg/Mem + (0.5 – X) for ALU + (0.2 – X) for Load + 0.1 for Store + 0.2 for branch) I = (1 – X) I, where I is the old instn count

CPInew = X . 2 + (0.5 – X) . 1 + (0.2 – X) . 2 + 0.1 . 3 + 0.2 . 2/ (1-X) = (1.6 – X) / (1-X).

To pay off, CPInew . Inew <= CPI old x I => ((1.6-X)/(1-X)) . (1-X) I <= 1.6 x I

Hence, X >= 0, ie. Any fraction of load that can be eliminated will perform better

(c) Design a new MIPS instruction format for this new Reg/Mem instruction. Explain how it is different compared to the existing R and I formats.                             ---- 6 pts

Use the same R-format for MIPS, but let the field rt represent the register, which stores the memory location. We can use shamt to denote offset or point to a register, where the offset is stored. Keep the same funct field to represent different functions and shift amount together.

The difference compared to the R-format is described above. It is different from the I format, given below. It has only two register fields, one for the memory address and another for the destination register.

2. (a) Consider a processor running at 1 GHz and a CPI of 0.7 excluding memory access. It has a set-associative cache of 32KB with 2 cycle access time. The cache is write-through with a 5% miss rate for data. The cache block size is 32 bytes. The memory has an access time of 60 ns and is connected to the cache by a data bus that can transfer 8 bytes of data per cycle. What are the average memory access times (AMAT) for data load, and data write operations?  How much would be the AMAT if it was a write-buffered cache?                                                                                                   ---- 8 pts

Answer: CPU cycle time for 1 GHz = 1 ns. Block transfer time = 32/8 = 4 cycles

Total memory access time = 60 + 32/8 = 64 ns.

AMAT Read = 2 + 0.05 . 64 = 5.2 ns (0.95 . 2 + 0.05 . 64) also correct.

AMT Write-through = 2 + 60 + 1 = 63 ns => Always writes a word to memory

AMAT Write-back = 2 ns because it only writes to cache and a buffer.

(b) Assume a main memory with 4096 blocks, a set associative cache with 2 sets and 2 blocks per set. A least recently used (LRU) policy is used for cache block replacement.

Show a table of contents of cache blocks for the following sequence of block addresses: 0,1,4,6,0,7,1,0,6,4. Indicate whether the access is a hit or miss.                        ----- 12 pts

Block id                   Set              Block              H/M

0                            0                   0                    M

1                            1                   0                    M

4                            0                   1                    M

6                            0                   0                    M

0                            0                   1                    M

7                            1                   1                    M

1                            1                   0                    H

0                            0                   1                    H

6                            0                   0                    H

4                            0                   1                    M

3. (a) Assume that a datapath has a cycle time of 7 ns. It is now split into 5 pipeline stages with IF =2 ns, ID = 1 ns, EX = 1 ns, MEM = 3 ns, and WB = 1 ns. Design a pipeline architecture, where each stage takes one cycle. What is the cycle time, CPI, and speed up of the pipelined machine? If there is stall every 4 instructions, what is the CPI of the new machine?                                                                                             ------ 6 pts

Answer: The MEM stage is the bottleneck. Hence the cycle time has to be 3 ns for a pipelined machine. Then the CPI = 1, Speed up = 7/3 = 2.33

If there is a stall once in every 4 instructions, the CPI = 5/4 = 1.25

(b) Assume you can have different cycles for different stages. How are the cycle time, CPI and speed-up changed?                                                                             ------ 6 pts

Answer: Given the time taken by different stages in (a), the cycle time = 1 ns.

However, an instruction will complete only once in 3 cycles due to MEM stage.

Hence CPI = 3. Speed up = 7/3 = 2.33 – remains same.

(c) Assume the following sequence of instructions being executed on the pipeline of 3(b). Draw the space-time diagram.                                                                        --------8 pts

LW R3, 0(R0), LW R1, 0(R3), ADDI R1, R1, #1, SUB R4, R3, R2, SW R1, 0(R3)

Answer: The sequence of instructions is being executed on pipeline of 3 (b), which takes 2 cycles for IF, 1 cycle for ID and EX, 3 cycles for MEM, and 1 cycle for WB.

Assuming no forwarding, the space-time diagram is given below.

(1) LW R3, 0(R0)          IF, IF, ID, EX, MEM, MEM, MEM, WB

(2) LW R1, 0(R3)                      IF,  IF,     X           X        X    ,  ID,  EX,  MEM, MEM, MEM, WB

(3) ADDI R1, R1, #1                                                                     IF,   IF,          X          X     X       ID, EX, MEM, MEM, MEM, WB

(4) SUB R4, R3, R2                                                                                                                             IF,  IF,    ID,      EX,    MEM, MEM, MEM, WB

(5) SW R1, 0(R3)                                                                                                                                                 IF,       IF,       X         ID,       EX,    MEM, MEM WB

Note: Dependency between  (1) and (2) delays ID of (2) until WB of (1). Similar case for (2) to (3). (4) is independent instruction, so execution proceeds without any stall. Dependence between (3) and (5), so ID is delayed until WB of (3)