# CS 203: Computer Architecture

Winter 2018, Test 1

# Points: 60 points                                                                      Time: 1 hour and 20 mins

1. (a) Assume that a computer at 1 GHz frequency is running a program with 20,000 instructions. The instruction mix consists of 50% ALU, 20% load, 10% store, and 20% branch instructions, which take 1, 2, 3 and 2 cycles, respectively. Derive the CPI of the program.                                                                                                               --- 6 pts

(b) Instead of a load-store architecture, assume that you would like to introduce a new Reg/Mem instruction that can get one operand from register and another from memory location for an arithmetic operation. What fraction of load must be eliminated for this instruction to pay off?                                                                                       ----- 8 pts

(c) Design a new MIPS instruction format for this new Reg/Mem instruction. Explain how it is different compared to the existing R and I formats.                             ---- 6 pts

2. (a) Consider a processor running at 1 GHz and a CPI of 0.7 excluding memory access. It has a set-associative cache of 32KB with 2 cycle access time. The cache is write-through with a 5% miss rate for data. The cache block size is 32 bytes. The memory has an access time of 60 ns and is connected to the cache by a data bus that can transfer 8 bytes of data per cycle. What are the average memory access times (AMAT) for data load, and data write operations?  How much would be the AMAT if it was a write-buffered cache?                                                                                                   ---- 8 pts

(b) Assume a main memory with 4096 blocks, a set associative cache with 2 sets and 2 blocks per set. A least recently used (LRU) policy is used for cache block replacement.

Show a table of contents of cache blocks for the following sequence of block addresses: 0,1,4,6,0,7,1,0,6,4. Indicate whether the access is a hit or miss.                        ----- 12 pts

3. (a) Assume that a datapath has a cycle time of 7 ns. It is now split into 5 pipeline stages with IF =2 ns, ID = 1 ns, EX = 1 ns, MEM = 3 ns, and WB = 1 ns. Design a pipeline architecture, where each stage takes one cycle. What is the cycle time, CPI, and speed up of the pipelined machine? If there is stall every 4 instructions, what is the CPI of the new machine?                                                                                             ------ 6 pts

(b) Assume you can have different cycles for different stages. How are the cycle time, CPI and speed-up changed?                                                                             ------ 6 pts

(c) Assume the following sequence of instructions being executed on the pipeline of 3(b). Draw the space-time diagram.                                                                        --------8 pts

LW R3, 0(R0), LW R1, 0(R3), ADDI R1, R1, #1, SUB R4, R3, R2, SW R1, 0(R3)