# CS 203A: Computer Architecture

Winter 2018, Test 2

# Points: 80                                                                                               Time: 2 hours

1. (a) What is Amdahl’s law? Assume that we have a new CPU with an enhanced ALU that can speed up the ALU operations by a factor of two. The instruction mix consists of 50% ALU, 30% load/store, and 20% branch instructions, each of which takes 2, 5, and 3 cycles, respectively. What is the speed-up for the new CPU at the 1 GHz frequency?

--- 8 pts

(b) Give the equations for the dynamic power and energy consumption in a single-core CPU. Consider a dual-core processor that can achieve the same execution time with 60% reduction in voltage and frequency. What is the fraction of energy saving for the same program using the dual-core CPU?                                                           ---6 pts

(c) Describe the operation of a 2-bit branch predictor. Explain how the prediction accuracy can be further improved with a correlating predictor.                             -- 6 pts

2. Consider the following program. Assume that there is one EX unit that does both the execution and address computation. The LD/ST operations take one more cycle for memory access during which the EX unit is free to execute another instruction. Branch slot delay is one cycle.

Loop:   LD	 R2, 0(R1)

DADDIU R2, R2, #1

SD	 R2, 0(R1)

DADDIU R1, R1, #8

BNEZ	R1, Loop

(a) Show the execution sequence with stalls for an in-order single issue pipeline.

----- 8 pts

(b) 1. Unroll the loop twice and rewrite the sequence of instructions, 2. Rearrange the instructions to reduce the stalls, and show the execution sequence.             ---- 12 pts.

(c) Assume that the above loop is executed using an in-order dual-issue out-of-order processor. The execution timing for 2 iterations is attached without speculation.

Assuming speculation and perfect branch prediction, fill the new timings in the empty diagram attached. Explain the changes compared to the original timing.     -----10 pts

3. (a) What is VLIW architecture? What are its advantages and disadvantages?  -- 7 pts

(b) What are different hazards in an out-of-order superscalar processor? How are they solved in Scoreboarding and Tomasulo’s architectures?                                    ----- 8 pts

(c) 1. Why and how is a CPU architecture needs modification for hardware multithreading?  Why? 2. Explain the difference between fine-grain and coarse-grain multithreading with their advantages/disadvantages?                                         ---- 8 pts

(d)  What is a cache coherence problem in shared memory multiprocessors? Briefly explain how is it solved in an SMP and in a DSM?                                              --- 7 pts

GOOD LUCK

3.(a) Execution timing WITHOUT speculation:

3.(b) Execution timing WITH speculation: